6 May 2025

MEMORANDUM FOR  RECORD

FROM:  75 ABW/SE

SUBJECT:  Major Mishap Response – SUAS Considerations

- The purpose of this memo is to distill a list of considerations for Safety, EOC (Emergency Operations Center), and CAT (Commanders Action Team) members to analyze before a major mishap such that DoD-controlled SUAS (Small Unmanned Aerial Systems) may be leveraged in support of Commanders and ISB (Interim Safety Board) / SIB (Safety Investigation Board) efforts.

- Personnel coordinating SUAS employment should consider the following items:

  - Asset Availability. Security Forces, Civil Engineering, Public Affairs, and Safety may possess various types of SUAS assets. Liaise with appropriate SMEs and/or the SUAS Program Manager. If a Concept of Employment (CONEMP) document is not signed, the wing commander will likely need to approve the use of SUAS.

  - Gameplan. Verify SUAS operators understand what they are looking for and any relevant time/altitude/flightpath restrictions. Verify adequate communication plan between C2, ATC (Air Traffic Control) and operator (radio, cell phone, etc).

  - Deconfliction. Ensuring time, geographic, and/or altitude deconfliction from manned systems is paramount. SUAS lighting, size, lost link protocols, ATC awareness, and visual monitoring are factors.

  - Airspace. An NDA (National Defense Authorization) and/or TFR (Temporary Flight Restriction) will likely be activated for major mishaps. Verify radius, altitude, clearance authorities and procedures for entering.

  - Applications.

    - Expediting collection of ISB evidence during time-critical scenarios like restoration of runway capacity.

    - Validating “plume model” predictions of weather effects on toxic smoke/materials.

    - Searching for critical components in limited-access areas (mountainous terrain, etc.)

  - Capabilities. SUAS may be configured with a wide range of capabilities. It is advantageous to collect evidence of the same scene/perspective through multiple spectrums.

    - EO: Electro Optical

    - IR: Infrared / thermal imaging.

    - NV: Night Vision

    - LiDAR: Light Detection and Ranging. Can create 3D maps and models of crash sites.

    - Battery Life. Loiter time may be very limited. Consider prioritizing objectives.

    - Wind limitations: extreme winds may make control difficult and reduce loiter time.

    - Weather limitations: battery life may be less than expected in periods of extreme temperature. Precipitation, low clouds, and smoke may be a factor.

  - Launch and Recovery. Verify location of launch and recovery does not interfere with other operations.

  - Anti-Drone systems. If operating within the space of counter-SUAS equipment, ensure operations are coordinated.

  - Low/No Light Ops. If night ops are not authorized, determine restrictions related to sunrise/sunset and beginning/end of civil twilight. Ensure drone is sufficiently illuminated. Verify SUAS Sensors are effective during low light/night operations. Consider the effect of thermal cross-over on data collection.

  - Security. Consider the security sensitivity of the crash site or area being observed. Understand the encryption level (or lack thereof) of the video feed/data link between SUAS and operator.

  - Privacy. Consult JAG regarding privacy considerations. DAFI 14-404 para 5.3.1. addresses considerations for intelligence oversight of Unmanned Aircraft Systems activities.

  - Bandwagon Effect. Civilians may be encouraged to operate unauthorized drones upon seeing DoD drones in the area. Mitigate with Security Forces countermeasures and public affairs messaging as applicable.

  - Data Transfer. If Security Classification is a concern, PA or XP may need to be present to validate recorded content before transferring to other systems. Ensure standalone laptop or other required equipment is available for effective custody transfer of data.

3.  Questions regarding this memo may be directed to Maj Melvin Murphrey, 75ABW/SEG, melvin.murphrey.1@us.af.mil, DSN 777-7233.

MELVIN E. MURPHREY III, Maj, USAF

Flight Safety Officer